X-CAVATE

CLI & Python API Reference

Version 2.0.0

Convert vascular network geometries into collision-free
3D printer toolhead pathways and G-code.

SM = Single-Material  |  MM = Multimaterial

This document combines content from the original X-CAVATE Supplement User Manual with new CLI and Python API documentation for the refactored package. Sections containing new text are marked with a purple "Text added by Claude" callout box.

# 1. Introduction

Text added by Claude: X-CAVATE (Cross-Collision Avoidance for VAscular Tree Embedding) converts vascular network geometries exported from SimVascular into collision-free 3D printer toolhead pathways and G-code. The software supports three usage modes:

1. Streamlit GUI — A web-based graphical interface requiring no coding experience. See the companion GUI User Guide for full documentation.
2. Command-Line Interface (CLI) — The xcavate command for scripted and batch workflows.
3. Python API — Import xcavate as a library for programmatic access.

This document serves as the reference manual for the CLI and Python API. It also preserves the theoretical background and workflow descriptions from the original supplement user manual.

# 2. Setup

## 2.1 Original Setup Instructions

Navigate to https://github.com/sohams-MASS/X-CAVATE. Click on the green "Code" button and download the .zip file. Place the downloaded folder in the directory of your choice, such as the Desktop.

Text added by Claude: The repository URL has changed from the original github.com/jessica-herrmann/xcavate to the current github.com/sohams-MASS/X-CAVATE. The folder structure has been refactored from a single-script layout into a proper Python package.

## 2.2 Installation

### Option A: Conda (Recommended)

1. Open a terminal (Mac: Terminal.app; Windows: Anaconda Prompt or PowerShell).
2. Confirm conda is installed:

conda --version

If not installed, download Miniconda from https://docs.conda.io/en/latest/miniconda.html.
3. Clone the repository and enter the directory:

git clone https://github.com/sohams-MASS/X-CAVATE.git
cd X-CAVATE

4. Create and activate the conda environment:

conda env create -f environment.yml
conda activate xcavate-gui

5. Verify the installation:

xcavate --help

### Option B: pip

1. Confirm Python 3.9+ is installed: python3 --version
2. Clone the repository (same as above).
3. Install with GUI support:

pip install ".[gui]"

For CLI-only usage (no web interface): pip install .

### Option C: Docker

1. Install Docker Desktop from https://www.docker.com/products/docker-desktop.
2. Build the image:

docker build -t xcavate .

3. Run the container:

docker run -p 8501:8501 xcavate

4. Open http://localhost:8501 in your browser.

# 3. Input Files

## 3.1 Inputs Overview

The inputs folder contains two subfolders: custom and extension.

### Extension Folder

For a description of how to utilize the inputs/extension folder to implement segment extension (gap closure), see the Gap Closure section of this guide.

### Custom G-code Folder

The inputs/custom folder contains sample .txt files which can be updated to contain custom G-code wrappers to adapt X-CAVATE to your specific printer. For example, to provide your own G-code for header information, replace the placeholder text within header_code.txt with your own code to accomplish this task.

## 3.2 Network File Format

Vessel: 0, Number of Points: 100
33.4878, 0.0000, 53.5303, 0.0200
33.4115, -0.5285, 52.7311, 0.0200
...
Vessel: 1, Number of Points: 100
15.0600, -37.3300, 11.6600, 0.0190
...

| Column | Required | Description |
| --- | --- | --- |
| 1–3 | Yes | x, y, z coordinates (cm) |
| 4 | No | Vessel radius (needed for speed calculation) |
| 5 | No | Arterial/venous flag: 0 = venous, nonzero = arterial (needed for multimaterial) |

Warning: Coordinates in the network file must be in centimeters — X-CAVATE converts to millimeters internally (multiplication by 10).

## 3.3 Inlet / Outlet File Format

inlet
21.97, -15.43, 50.00
11.81, 0.00, 21.34
outlet
36.05, -27.19, 50.00
50.00, 0.00, 26.54

Coordinates are matched to the nearest network point automatically.

# 4. Running X-CAVATE via CLI

Text added by Claude: The original workflow required typing python3 xcavate.py followed by all parameters, and optionally using the command_line_writer.py helper script to generate the command string. In the refactored package, X-CAVATE is installed as a console entry point. The command is simply xcavate. The command_line_writer.py script is no longer needed — all parameters are passed directly as CLI flags.

## 4.1 Basic Invocation

xcavate \
  --network_file inputs/network.txt \
  --inletoutlet_file inputs/inlet_outlet.txt \
  --multimaterial 0 \
  --tolerance_flag 0 \
  --nozzle_diameter 0.5 \
  --container_height 10 \
  --num_decimals 3 \
  --speed_calc 0 \
  --plots 1 \
  --downsample 0 \
  --custom 0 \
  --printer_type 0

## 4.2 Complete CLI Argument Reference

Text added by Claude: The following tables document every CLI argument. They are derived directly from the argument parser in xcavate/cli.py.

### Required Arguments

| Flag | Type | Default | Description |
| --- | --- | --- | --- |
| --network_file | str | — | Path to .txt file containing network coordinates. |
| --inletoutlet_file | str | — | Path to .txt file containing inlet and outlet coordinates. |
| --multimaterial | int | — | Multimaterial mode (1=yes, 0=no). |
| --tolerance_flag | float | 0 | Include tolerance? (1=yes, 0=no). |
| --tolerance | float | 0 | Amount of tolerance (0 is none). |
| --nozzle_diameter | float | — | Outer diameter of the nozzle (mm). |
| --container_height | float | — | Height of the print container (mm). |
| --num_decimals | int | — | Number of decimal places for rounding output values. |
| --speed_calc | int | — | Compute print speeds for changing radii? (1=yes, 0=no). |
| --plots | int | — | Generate plots? (1=yes, 0=no). |
| --downsample | int | — | Downsample network? (1=yes, 0=no). |
| --custom | int | — | Providing custom G-code? (1=yes, 0=no). |
| --printer_type | int | 0 | Type of printer: 0=Pressure, 1=Positive Ink, 2=Aerotech. |
| --amount_up | float | 10 | Z-raise above container for nozzle switches (mm). |

### Geometry Parameters

| Flag | Type | Default | Description |
| --- | --- | --- | --- |
| --container_x | float | 50 | Container x-dimension (mm). |
| --container_y | float | 50 | Container y-dimension (mm). |
| --scale_factor | float | 1 | Network scale factor. |
| --top_padding | float | 0 | Padding above network (mm). |
| --downsample_factor | int | 1 | Downsample factor. |

### Print Settings

| Flag | Type | Default | Description |
| --- | --- | --- | --- |
| --flow | float | 0.161 | Volumetric flow rate (mm³/s). |
| --print_speed | float | 1 | Print speed (mm/s). |
| --jog_speed | float | 5 | Jog speed (mm/s). |
| --jog_speed_lift | float | 0.25 | Z-lift jog speed (mm/s). |
| --initial_lift | float | 0.5 | Initial lift distance (mm). |
| --jog_translation | float | 10 | Jog speed between nozzles (mm/s). |
| --dwell_start | float | 0.08 | Dwell at start (s). Aerotech only. |
| --dwell_end | float | 0.08 | Dwell at end (s). Aerotech only. |

### Multimaterial Settings

| Flag | Type | Default | Description |
| --- | --- | --- | --- |
| --offset_x | float | 103 | Printhead x-offset (mm). |
| --offset_y | float | 0.5 | Printhead y-offset (mm). |
| --front_nozzle | int | 1 | 1 if venous nozzle in front, 2 if behind. |
| --printhead_1 | str | Aa | Arterial printhead name. |
| --printhead_2 | str | Ab | Venous printhead name. |
| --axis_1 | str | A | Arterial z-axis name. |
| --axis_2 | str | B | Venous z-axis name. |
| --resting_pressure | float | 0 | Resting pressure (psi). |
| --active_pressure | float | 5 | Active pressure (psi). |

### Gap Closure

| Flag | Type | Default | Description |
| --- | --- | --- | --- |
| --num_overlap | int | 0 | Overlap nodes at pass boundaries. |
| --overlap_algorithm | str | retrace | Overlap algorithm: retrace (original). |
| --close_sm | int | 0 | Load gap closure file for SM? (1=yes, 0=no). |
| --close_mm | int | 0 | Load gap closure file for MM? (1=yes, 0=no). |

### Positive Ink Displacement

| Flag | Type | Default | Description |
| --- | --- | --- | --- |
| --positiveInk_start | float | 0 | Plunger displacement at pass start. |
| --positiveInk_end | float | 0 | Plunger displacement at pass end. |
| --positiveInk_radii | int | 0 | Use per-node vessel radii? (1=yes, 0=no). |
| --positiveInk_diam | float | 1 | Fixed vessel diameter (mm). |
| --positiveInk_syringe_diam | float | 1 | Syringe barrel diameter (mm). |
| --positiveInk_factor | float | 1 | Extrusion scaling factor. |
| --positiveInk_start_arterial | float | 0 | Arterial plunger displacement at start. |
| --positiveInk_start_venous | float | 0 | Venous plunger displacement at start. |
| --positiveInk_end_arterial | float | 0 | Arterial plunger displacement at end. |
| --positiveInk_end_venous | float | 0 | Venous plunger displacement at end. |

### Advanced Flags

Text added by Claude: These flags were added in the refactored version and were not present in the original xcavate.py script.

| Flag | Type | Default | Description |
| --- | --- | --- | --- |
| --algorithm | str | dfs | Pathfinding algorithm: dfs (depth-first search). |
| --output_dir | str | outputs | Output directory path. |

## 4.3 Example Commands

Text added by Claude: The examples below are adapted from the original supplement user manual, updated for the new xcavate CLI entry point.

### Minimal Single-Material Run

xcavate \
  --network_file inputs/network.txt \
  --inletoutlet_file inputs/inlet_outlet.txt \
  --multimaterial 0 --tolerance_flag 0 \
  --nozzle_diameter 0.65 --container_height 50 \
  --num_decimals 5 --amount_up 10 \
  --speed_calc 0 --plots 1 --downsample 0 \
  --custom 0 --printer_type 0

### Multimaterial with Speed Calculation and Custom G-code

xcavate \
  --network_file inputs/multimaterial_network.txt \
  --inletoutlet_file inputs/inlet_outlet_multimaterial.txt \
  --multimaterial 1 --tolerance_flag 0 --tolerance 0 \
  --nozzle_diameter 0.65 --num_decimals 5 \
  --amount_up 10 --container_height 50 \
  --speed_calc 1 --plots 1 --downsample 0 \
  --flow 0.127 --custom 1 --printer_type 0

### Using a Custom Output Directory

xcavate \
  --network_file inputs/network.txt \
  --inletoutlet_file inputs/inlet_outlet.txt \
  --multimaterial 0 --tolerance_flag 0 \
  --nozzle_diameter 0.5 --container_height 10 \
  --num_decimals 3 --speed_calc 0 --plots 1 \
  --downsample 0 --custom 0 --printer_type 0 \
  --output_dir my_results

# 5. Running X-CAVATE via Python API

Text added by Claude: This entire section documents the Python API introduced in the refactored package. It enables programmatic access to X-CAVATE for scripting, batch processing, and integration into larger workflows.

## 5.1 Quick Start

from pathlib import Path
from xcavate.config import XcavateConfig
from xcavate.pipeline import run_xcavate

config = XcavateConfig(
    network_file=Path("inputs/network.txt"),
    inletoutlet_file=Path("inputs/inlet_outlet.txt"),
    nozzle_diameter=0.5,
    container_height=10.0,
    num_decimals=3,
)

results = run_xcavate(config)

# Access results
print(f"SM passes: {len(results['print_passes_sm'])}")
print(f"Output in: {config.output_dir}/")

## 5.2 XcavateConfig Dataclass

The XcavateConfig dataclass (defined in xcavate/config.py) replaces the 30+ global variables from the original script. All fields with defaults are optional.

### Required Parameters

| Field | Type | Default | Description |
| --- | --- | --- | --- |
| network_file | Path | — | Path to network coordinates file. |
| inletoutlet_file | Path | — | Path to inlet/outlet file. |
| nozzle_diameter | float | — | Outer diameter of nozzle (mm). |
| container_height | float | — | Height of print container (mm). |
| num_decimals | int | — | Decimal places for output rounding. |

### Feature Flags

| Field | Type | Default | Description |
| --- | --- | --- | --- |
| amount_up | float | 10.0 | Z-raise between passes (mm). |
| multimaterial | bool | False | Enable multimaterial mode. |
| tolerance_flag | bool | False | Enable tolerance. |
| speed_calc | bool | False | Compute per-node speeds from radii. |
| generate_plots | bool | True | Create 3D HTML visualizations. |
| downsample | bool | False | Enable downsampling. |
| custom_gcode | bool | False | Use custom G-code templates. |
| printer_type | PrinterType | PRESSURE | Printer type enum. |
| algorithm | PathfindingAlgorithm | DFS | Pathfinding strategy enum. |

### Geometry

| Field | Type | Default | Description |
| --- | --- | --- | --- |
| tolerance | float | 0.0 | Tolerance distance (mm). |
| scale_factor | float | 1.0 | Network scale multiplier. |
| top_padding | float | 0.0 | Padding above network top (mm). |
| container_x | float | 50.0 | Container width (mm). |
| container_y | float | 50.0 | Container depth (mm). |
| convert_factor | float | 10.0 | Unit conversion (cm → mm). |
| downsample_factor | int | 1 | Downsample factor. |

### Print Settings

| Field | Type | Default | Description |
| --- | --- | --- | --- |
| flow | float | 0.161 | Volumetric flow rate (mm³/s). |
| print_speed | float | 1.0 | Print speed (mm/s). |
| jog_speed | float | 5.0 | Jog speed (mm/s). |
| jog_speed_lift | float | 0.25 | Z-lift jog speed (mm/s). |
| jog_translation | float | 10.0 | Inter-nozzle jog speed (mm/s). |
| initial_lift | float | 0.5 | Initial lift distance (mm). |
| dwell_start | float | 0.08 | Dwell at start (s). |
| dwell_end | float | 0.08 | Dwell at end (s). |

### Multimaterial

| Field | Type | Default | Description |
| --- | --- | --- | --- |
| offset_x | float | 103.0 | Printhead x-offset (mm). |
| offset_y | float | 0.5 | Printhead y-offset (mm). |
| front_nozzle | int | 1 | 1=venous in front, 2=behind. |
| printhead_1 | str | "Aa" | Arterial printhead name. |
| printhead_2 | str | "Ab" | Venous printhead name. |
| axis_1 | str | "A" | Arterial z-axis name. |
| axis_2 | str | "B" | Venous z-axis name. |
| resting_pressure | float | 0.0 | Resting pressure (psi). |
| active_pressure | float | 5.0 | Active pressure (psi). |

### Gap Closure

| Field | Type | Default | Description |
| --- | --- | --- | --- |
| num_overlap | int | 0 | Overlap nodes at boundaries. |
| overlap_algorithm | OverlapAlgorithm | RETRACE | Overlap algorithm enum. |
| close_sm | bool | False | Load SM gap closure file. |
| close_mm | bool | False | Load MM gap closure file. |

### Positive Ink Displacement

| Field | Type | Default | Description |
| --- | --- | --- | --- |
| positive_ink_start | float | 0.0 | Plunger displacement at pass start. |
| positive_ink_end | float | 0.0 | Plunger displacement at pass end. |
| positive_ink_radii | bool | False | Use per-node vessel radii. |
| positive_ink_diam | float | 1.0 | Fixed vessel diameter (mm). |
| positive_ink_syringe_diam | float | 1.0 | Syringe barrel diameter (mm). |
| positive_ink_factor | float | 1.0 | Extrusion scaling factor. |
| positive_ink_start_arterial | float | 0.0 | Arterial start displacement. |
| positive_ink_start_venous | float | 0.0 | Venous start displacement. |
| positive_ink_end_arterial | float | 0.0 | Arterial end displacement. |
| positive_ink_end_venous | float | 0.0 | Venous end displacement. |

### Paths

| Field | Type | Default | Description |
| --- | --- | --- | --- |
| custom_gcode_dir | Path | inputs/custom | Directory for custom G-code templates. |
| extension_dir | Path | inputs/extension | Directory for gap extension files. |
| output_dir | Path | outputs | Output directory. |

## 5.3 Enums

from xcavate.config import PrinterType, PathfindingAlgorithm, OverlapAlgorithm

# PrinterType
PrinterType.PRESSURE       # 0 - Pneumatic extrusion
PrinterType.POSITIVE_INK   # 1 - Positive ink displacement
PrinterType.AEROTECH       # 2 - Aerotech 6-axis controller

# PathfindingAlgorithm
PathfindingAlgorithm.DFS   # "dfs" - Modified depth-first search
# Only DFS is currently supported

# OverlapAlgorithm
OverlapAlgorithm.RETRACE   # "retrace" - Scan all previous passes

## 5.4 run_xcavate() Function

def run_xcavate(
    config: XcavateConfig,
    progress_cb: Optional[Callable[[int, str], None]] = None,
) -> dict:

Parameters:

config — Fully populated XcavateConfig instance.

progress_cb — Optional callback (step_number, description) for progress reporting. Steps are numbered 1–7.

Returns a dict with the following keys:

| Key | Description |
| --- | --- |
| print_passes_sm | Single-material print passes (dict of pass index → node list). |
| print_passes_mm | Multimaterial print passes (or None). |
| points | Interpolated coordinate array (numpy ndarray). |
| points_original | Original coordinate array before interpolation. |
| coord_num_dict_original | Original vessel → point count mapping. |
| changelog | List of changelog lines from gap closure. |
| speed_map_sm | Speed map for SM (or None). |
| speed_map_mm | Speed map for MM (or None). |
| material_map | Material classification per pass (or None). |
| instructions | Operator instructions dict (start position, padding, etc.). |

## 5.5 Complete Example

from pathlib import Path
from xcavate.config import (
    XcavateConfig,
    PrinterType,
    PathfindingAlgorithm,
)
from xcavate.pipeline import run_xcavate

config = XcavateConfig(
    network_file=Path("inputs/multimaterial_network.txt"),
    inletoutlet_file=Path("inputs/inlet_outlet_mm.txt"),
    nozzle_diameter=0.65,
    container_height=50.0,
    num_decimals=5,
    multimaterial=True,
    speed_calc=True,
    flow=0.127,
    generate_plots=True,
    custom_gcode=True,
    printer_type=PrinterType.PRESSURE,
    algorithm=PathfindingAlgorithm.DFS,
    output_dir=Path("my_output"),
)

results = run_xcavate(config)

# Inspect results
sm = results["print_passes_sm"]
mm = results["print_passes_mm"]
print(f"Generated {len(sm)} SM passes")
if mm:
    print(f"Generated {len(mm)} MM passes")

# Access start position for operator instructions
instr = results["instructions"]
pos = instr["start_position"]
print(f"Start at X={pos['x']}, Y={pos['y']}, Z={pos['z']}")

# 6. Calibration

## 6.1 Updating Vessel Radii

Follow the calibration protocol to create diameters.csv and diameters_validation.csv. In the original workflow, these files were placed within the xcavate-main/inputs folder and processed by running python3 calibration.py from the command line. The computed volumetric flow rate, Q, was output to the command line interface and saved within a subfolder xcavate-main/outputs/radii.

Text added by Claude: In the refactored version, calibration is performed through either:
• The Calibration tab in the Streamlit GUI (see GUI User Guide).
• The Python API functions described below.

The standalone calibration.py and calibration_validation.py scripts have been replaced by the xcavate.core.calibration module.

## 6.2 Q Computation Equations

To incorporate the specified vessel radii, X-CAVATE continuously updates the print speed of each coordinate within the network, at constant pressure, to dynamically alter the printed filament diameter. Computation of the print speed depends on the following definition of volumetric flow rate:

Equation 1:  Q = V / t

where Q is the volumetric flow rate of the ink through the nozzle orifice, V is the volume of the printed filament, and t is the duration of the print. Assuming that the cylindrical nozzle extrudes a cylindrical filament, V becomes πr²L, where r is the filament radius and L is the length of the printed filament.

Equation 2:  Q = πr²L / t

Since L is equivalent to the nozzle translation velocity, v, multiplied by the duration of travel, t:

Equation 3:  Q = πr²vt / t

Rearrangement results in:

Equation 4:  r = √(Q / πv)

Thus, for a known filament radius r and ink volumetric flow rate Q, the nozzle translation velocity v can be calculated.

## 6.3 Python API: Calibration Functions

Text added by Claude: The calibration functions below are defined in xcavate/core/calibration.py and can be called directly from Python.

### compute_flow_rate()

from xcavate.core.calibration import compute_flow_rate

result = compute_flow_rate(
    measured_csv_bytes=open("diameters.csv", "rb").read(),
    print_speeds=[0.5, 1.0, 2.0],     # mm/s, one per group
    measurements_per_target=3,          # rows to average per group
    output_dir=Path("outputs/radii"),   # optional: save to disk
)

print(f"Q = {result['q_value']:.4f} mm^3/s")
# result also contains: 'figure', 'q_txt_bytes', 'html_bytes'

Parameters:

measured_csv_bytes — Raw bytes of a CSV file with a Length column containing measured filament diameters in microns.

print_speeds — Print speeds in mm/s used during calibration, one per group. Pressure is held constant; speed is varied.

measurements_per_target — Number of rows in the CSV to average for each print speed group (default: 3).

output_dir — If provided, Q_pressure.txt and radii_pressure.html are written to this directory.

Returns a dict with keys: q_value (float), figure (Plotly Figure), q_txt_bytes (bytes), html_bytes (bytes).

### validate_calibration()

from xcavate.core.calibration import validate_calibration

result = validate_calibration(
    target_diameters=[0.5, 0.8, 1.0],   # expected diameters in mm
    measured_csv_bytes=open("diameters_validation.csv", "rb").read(),
    measurements_per_target=3,
)

for t, m, e in zip(
    result["target"], result["measured_avg"], result["percent_error"]
):
    print(f"Target: {t} mm, Measured: {m} mm, Error: {e}%")
# result also contains: 'csv_bytes', 'html_bytes', 'figure'

Parameters:

target_diameters — Expected diameters in mm.

measured_csv_bytes — Raw bytes of a CSV file with a Length column containing measured diameters in microns.

measurements_per_target — Number of rows to average per target diameter (default: 3).

Returns a dict with keys: target, measured_avg, percent_error (lists), csv_bytes (bytes), html_bytes (bytes), figure (Plotly Figure).

# 7. Outputs

## 7.1 Output Overview

The output from X-CAVATE automatically saves within the outputs folder (or the directory specified by --output_dir), which, prior to running X-CAVATE, is empty. Output takes the form of G-code files, network plots, the mathematical graph describing the network, and a changelog.

## 7.2 Directory Structure

outputs/
  graph/
    graph.txt               # Adjacency graph
    special_nodes.txt       # Branchpoints, endpoints, inlets, outlets
    SM_x_coords.txt         # Single-material x coordinates per pass
    SM_y_coords.txt         # Single-material y coordinates per pass
    SM_z_coords.txt         # Single-material z coordinates per pass
    SM_combined.txt         # Combined xyz per pass
    MM_x_coords.txt         # Multimaterial x coordinates (if enabled)
    MM_y_coords.txt         # Multimaterial y coordinates (if enabled)
    MM_z_coords.txt         # Multimaterial z coordinates (if enabled)
    MM_combined.txt         # Multimaterial combined xyz (if enabled)
  gcode/
    gcode_SM_pressure.txt   # Single-material G-code
    gcode_MM_pressure.txt   # Multimaterial G-code (if enabled)
  plots/
    network_original.html   # 3D plot of original network
    network_SM.html         # 3D plot of SM print passes
    network_MM.html         # 3D plot of MM print passes (if enabled)
  changelog.txt             # Gap closure operations log

Info: The G-code filename suffix matches your selected printer type: pressure, positive_ink, or aerotech.

## 7.3 Output File Table

| File | Directory | Description |
| --- | --- | --- |
| graph.txt | graph/ | List of nodes and edges constituting the mathematical graph describing the network. |
| special_nodes.txt | graph/ | Node numbers corresponding to special nodes within the graph (branchpoints, endpoints, inlets, outlets, daughter nodes). |
| SM_x_coords.txt | graph/ | Cartesian x-coordinates grouped by print pass and print order (single material). |
| SM_y_coords.txt | graph/ | Cartesian y-coordinates grouped by print pass (single material). |
| SM_z_coords.txt | graph/ | Cartesian z-coordinates grouped by print pass (single material). |
| SM_combined.txt | graph/ | List of Cartesian coordinates grouped by individual print pass for single material. |
| MM_*.txt | graph/ | Multimaterial equivalents of the above (if enabled). |
| gcode_SM_*.txt | gcode/ | Single-material G-code for the selected printer type. |
| gcode_MM_*.txt | gcode/ | Multimaterial G-code (if enabled). |
| network_original.html | plots/ | Interactive 3D visualization of the original vascular network. |
| network_SM.html | plots/ | Interactive 3D visualization of SM print passes. |
| network_MM.html | plots/ | Interactive 3D visualization of MM print passes. |
| changelog.txt | root | Log of all gap closure operations performed. |

# 8. Gap Closure / Segment Extension

## 8.1 Identifying Print Passes for Segment Extension

To identify an individual print pass for segment extension, open the network plot after running X-CAVATE. Hover over the endpoint node of interest; an informational box will appear which contains the x, y, z coordinates of the node, along with the print path (e.g., trace 35 refers to print pass 35).

To create additional extensions for single material printing, replace the placeholder text within pass_to_extend_SM.txt and deltas_to_extend_SM.txt. Use the pass_to_extend_SM.txt file to identify the numbers of the print passes, one per line, containing the endpoints to extend. Use the deltas_to_extend_SM.txt to specify the linear distance by which to extend the x-, y-, and z-coordinates of the endpoint. The process is the same for the multimaterial (_MM) files.

# 9. X-CAVATE Structure: Description of Network Processing

## 9.1 Pre-processing

X-CAVATE accepts an input vascular network in the form of two text (.txt) files. One of these files specifies the network in terms of the Cartesian coordinates composing its vessel splines, and the other specifies the coordinates of the inlets and outlets. The first text file, named "VascularNetwork.txt," consists of a minimum of 3 columns and a maximum of 5 columns. Column 1 contains the x-coordinates, column 2 contains the y-coordinates, and column 3 contains the z-coordinates; these three columns are mandatory. Column 4, if included, contains the radii measurements. Column 5, if included, contains the assignment as either arterial (represented with a "0") or venous (represented with a "1").

Upon importation of the network, X-CAVATE reformats the input file content into a graph. In a graph, coordinates termed "nodes" are connected to one another by lines, called "edges." Any two nodes which are connected to one another by one edge are termed "neighbors." Each node is also its own neighbor.

To label each coordinate with a nodal identity, the coordinates within the network are consecutively assigned whole numbers beginning with "0" until all nodes within the network have been assigned a number. Node types include "endpoint" nodes, "parent branch point" nodes, "daughter" nodes, and "standard" nodes.

Prior to generating the graph, X-CAVATE performs a linear interpolation to ensure that no adjacent coordinates within individual vessels are separated by a distance greater than the nozzle radius. This step introduces additional coordinates to allow the nodes to approximate the edges, which assists with collision detection during the modified depth-first search portion of the algorithm.

## 9.2 Modified Depth-First Search (DFS)

Following interpolation and structuring of the graph, X-CAVATE performs a modified depth-first search (DFS). DFS is a standard algorithm for traversing all nodes within a graph. Initially, all nodes within the graph are marked as "unvisited." To explore the graph, DFS begins from an initial node and traverses one of its connected edges to a neighboring node, repeating until reaching a node whose neighbors have all been visited. When this occurs, DFS backtracks until it encounters a node with an unvisited neighbor.

X-CAVATE adapts DFS to identify collision-free traversal of vascular networks. The first modification is the introduction of a validity function, which determines whether a node is "invalid" — one that, if printed, would result in an eventual collision. To identify invalid coordinates, X-CAVATE constructs a cylindrical boundary region around the node matching the nozzle cross-section dimensions, extending infinitely downward in the z-direction. If any unvisited nodes fall within this boundary, the current node is marked invalid, concluding the current "print pass" and triggering a new iteration starting at the positionally lowest unvisited node.

## 9.3 Post-processing

X-CAVATE incorporates post-processing steps to handle two features of the DFS algorithmic output. First, because the DFS algorithm incorporates backtracking, it occasionally traverses non-neighboring nodes. X-CAVATE subdivides the output print passes at these instances. Second, although the DFS ensures traversal of all nodes, it does not guarantee traversal of all edges. Non-traversed edges manifest in physical gaps within the printed ink filaments. X-CAVATE includes post-processing steps to sequentially check the first and last nodes of each print pass for unconnected neighbors and close these gaps.

## 9.4 Radius Computation

To incorporate the specified vessel radii, X-CAVATE continuously updates the print speed of each coordinate within the network, at constant pressure, to dynamically alter the printed filament diameter. The computation of the print speed depends on the volumetric flow rate equations presented in the Calibration section.

## 9.5 Generation of Print Passes for Multimaterial Printing

X-CAVATE enables users to specify which of the input vessels belong to the arterial circulation and which are venous. To translate the programmed vessel type to the 3D printed model, X-CAVATE generates print paths for multi-material printing with two distinct inks: arterial ink and venous ink. Generation occurs only if the VascularNetwork.txt file contains a fifth column specifying the vessel types. X-CAVATE further subdivides the post-processed print passes by vessel type, and G-code is used to incorporate nozzle translations between subdivided print passes to physically switch inks.

# 10. Module Structure

Text added by Claude: The original X-CAVATE codebase consisted of a single monolithic script (xcavate.py, approximately 5,568 lines) plus standalone calibration scripts. The refactored version is organized as a Python package with the following structure:

xcavate/
  __init__.py              # Package init
  __main__.py              # python -m xcavate entry point
  cli.py                   # Argument parser and CLI entry point
  config.py                # XcavateConfig dataclass and enums
  pipeline.py              # run_xcavate() orchestrator

  core/
    calibration.py          # compute_flow_rate(), validate_calibration()
    gap_closure.py          # Gap closure pipeline
    graph.py                # Graph construction and special node detection
    multimaterial.py        # Material classification and subdivision
    pathfinding.py          # DFS with KD-tree collision detection
    postprocessing.py       # Subdivision, downsampling, overlap, reordering
    preprocessing.py        # Interpolation and vessel boundary detection

  io/
    reader.py               # Network and inlet/outlet file parsing
    writer.py               # Coordinate output file writing
    gcode/
      base.py               # Base G-code writer and custom code loading
      pressure.py           # Pressure printer G-code writer
      positive_ink.py       # Positive ink displacement G-code writer
      aerotech.py           # Aerotech 6-axis G-code writer

  gui/
    app.py                  # Streamlit GUI application

  spatial/
    (spatial indexing utilities)

  viz/
    plotting.py             # Plotly 3D visualization functions

Text added by Claude: Each module corresponds to a logical stage of the original monolithic script:

• cli.py — Replaces the argparse block from xcavate.py lines 41–99 and the command_line_writer.py helper.
• config.py — Replaces the 30+ global variables from xcavate.py lines 101–171.
• pipeline.py — The orchestrator that wires all modules together.
• core/calibration.py — Replaces calibration.py and calibration_validation.py.
• core/pathfinding.py — Replaces the DFS implementation from the original script.
• io/gcode/ — Replaces the G-code generation sections, with separate writers for each printer type.